SONIC CLI

Contents

SONIC CLI	1

1.	Requirements	2

2.	Major design	2

2.1.	How to reuse current CLI?	3

2.2.	Support all commands of Quagga/FRR	3

2.3.	Different between SONIC CLI and VTYSH	5

2.4.	Authorization	5

Requirements

- Many questions about how to config.

- Tacacs+ authorization.

- Like traditional CLI to support:

Command completion by TAB, ? Help.

Configure mode, Interface mode, BGP mode.

- No conflict with current SONIC commands and easy to reuse.

Major design

ConfigDB

Quagga/FRR

Linux CLI

SONIC CLI

Host

BGP Docker

VTYSH

socket

APP Docker

Enforcer

APP

Tacacs+ Server

authorization

- SONIC CLI is running on host environment.

- Wrapping on current CLI to reuse it.

- SONIC CLI is developing based on VTYSH and support all commands of Quagga or FRR.

- Do command authorization by remote Tacacs+ server.

How to reuse current CLI?

VTYSH has a good defined framework and easily to create a new command. SONIC CLI just call linux command by a pipe.

Example:

- call existing SONIC comand

DEFUN (show_interface_counters,

show_interface_counters_cmd,  ---> function name

"show interface counters",      ---> command format

SHOW_STR                 ---> help string

INTERFACE_STR

"interfaces counters\n")

{

execute_linux_cmd_pipe("show interface counters -a");

return CMD_SUCCESS;

}

- call linux command

DEFUN (show_timezone,

show_timezone_cmd,

"show timezone",

SHOW_STR

"timezone\n")

{

execute_linux_cmd_pipe("timedatectl status");

return CMD_SUCCESS;

}

- call existing SONIC command to config

DEFUN (cli_aaa_auth_login_one,

aaa_auth_login_one_cmd,

"aaa authentication login (local|tacacs+)",

AAA_HELP_STR

AAA_AUTHENTICATION_HELP_STR

AAA_LOGIN_HELP_STR

"Local authentication\n"

"TACACS+ authentication\n")

{

EXEC_CMD_WITH_PARAM("sudo config aaa authentication login %s", argv[0]);

if(author_update_config())

vty_out(vty, "Update TACACS+ configuration fail\n");

return CMD_SUCCESS;

}

Support all commands of Quagga/FRR

- Compiling Quagga to generate a file named vtysh_cmd.c under src/sonic-quagga/vtysh/

- Copy it to src/sonic-cli/vtysh/

- Get all commands of Quagga

What CLI actually has been done with Quagga?

- CLI use vtysh_cmd.c to get the command tree from Quagga.

DEFSH (VTYSH_ZEBRA, show_ip_route_prefix_cmd_vtysh,

"show ip route A.B.C.D/M",

"Show running system information\n"

"IP information\n"

"IP routing table\n"

"IP prefix <network>/<length>,  e.g.,  35.0.0.0/8\n")

- CLI connects to vtysh server of zebra and bgpd.

- CLI send command to zebra. zebra will execute the command and return result to CLI.

- CLI printout the result.

bgpd

SONIC CLI

BGP Docker

VTYSH Server

socket

VTYSH Client

zebra

VTYSH Server

socket

1. Get a command

2. Send command

4. Return result

3. Execute command

5. Print result

Different between SONIC CLI and VTYSH

- SONIC CLI is an independent repository and can work without Quagga/FRR.

- SONIC CLI is running in host linux and easily to call existing python CLI and Linux commands.

- SONIC CLI supports authentication and authorization.

Authorization

- User enters to SONIC CLI by default when logins to switch.

/etc/passwd

yongcan.wyc:x:1003:1000:remote_user:/home/yongcan.wyc:/usr/bin/cli

- Get privilege level after do authentication by Tacacs+ server.

- Create a command tree based on privilege level.

Admin user gets a tree of all commands.

Other user gets a tree without high risk commands. For example, there is no command to enter configure mode, also no command to enter to Linux.

- Before executing any command, SONIC CLI will send it to Tacacs+ server to see if user has authorization to do it.

- All users including no admin user join to the sudo group and docker group, as some show commonds need sudo or docker. So if user passes authorization by Tacacs+, it can show anything successfully.